Please refer to PART 1: VORTEX GENERAL CLAUSES for all Play Product construction and installation information.

- Play Product Specifications:

  - Play Product Structure: The Spray Loop VOR-519.2008 shall be constructed of 304/304L stainless steel structural tubing with an outside diameter of 3½" (8.9cm) and a wall thickness of 0.120" (3mm). The loop shall be constructed of one continuous piece of tubing rolled to no less than a 72" (183cm) diameter with no joints or ripples.  The loop shall be mounted on a reinforced base plate to facilitate installation.  The SAFESWAP™ anchoring and leveling system shall be used.

  - Overall play product dimensions: The above ground height shall be 64" (162 cm).

  - Play Product Interactivity: Users can enjoy the spray mist from all directions inside the loop.

  - Hydraulic Activity/Components: The loop shall have five (5) stainless steel nozzles  that produce misty water effect.

  - Hydraulic Requirements: The combined hydraulic requirements of all five (5) spray nozzles shall be 5-10 gpm (18-38 lpm) @ 10-25 psi (0.7-1.7 bar).  The nozzle can be set in a -high flow- position in the case of water recirculation system to prevent clogging.