Please refer to PART 1: VORTEX GENERAL CLAUSES for all Play Product construction and installation information.

- Play Product Specifications:

  - Play Product Structure: The Water Jelly No. 1 VOR-7010.0002 shall be constructed of 304/304L stainless steel structural tubing with an outside diameter of 4½" (11.4cm) and a wall thickness of .120" (3mm). The brass spray head shall be threaded into the spray head body using a tamper-resistant tool. Tamper resistant brass winter cap shall be included. The Embedded anchoring and leveling system shall be used.

  - Overall play product dimensions: The overall height of the structure shall be no less than 5" (13cm)

  - Play Product Interactivity: Creates a soft, laminar spray with a unique texture that even young users can touch. A cone of water starts small and gently grows out and flows over in a soft, inviting cascade.

  - Hydraulic Activity/Components: The spray head shall produce an inverted laminar bell spray effect.

  - Hydraulic Requirements: The hydraulic requirements shall be 5-7 gpm ( 19-38 lpm) @ 3-5 psi (0.2 – 0.3 bar).