Please refer to PART 1: VORTEX GENERAL CLAUSES for all Play Product construction and installation information.

- Play Product Specifications:

  - Play Product Structure: The Flower No. 6 shall be constructed of 304/304L stainless steel structural tubing with an outside diameter of 3½" (8.9cm) and a wall thickness of 120" (3mm), with one (1) side arms constructed of 2½” (64mm) outside diameter 304/304L stainless steel structural tubing with wall thickness of .120” (3mm). On each end of the tubes there is one dumping flower and one spinner wheel. The flower bucket shall consist of a translucent SEEFLOW™ polymer piece thermoformed into the dumping flower shape. The dumping flower support shall be constructed of a stainless steel cap welded to a bended tube that will work as the counterweight of the flower bucket. The dumping flower shall be secured to the main post structure and the two arms with a stainless steel shaft and associated tamper-resistant hardware. The spinner wheel shall have a diameter of 30” (76cm) and shall be constructed of translucent SEEFLOW™ polymer with two Ultra High Molecular Weight Polyethylene center cones. The spinner wheel shall be secured to the main post structure with a stainless steel shaft and associated tamper resistant hardware.The SAFESWAP™ anchoring and leveling system shall be used.

  - Overall play product dimensions: The overall height of the Play Product shall be 153" (390 cm) above ground.

  - Play Product Interactivity:  Users can enjoy the reflection of sunlight and water on the spinning translucent water flower and try to predict when the dumping flower will fill and tip.

  - Hydraulic Activity/Components: The fill spout shall be constructed of 1.315” (33mm) outside diameter 304/304L stainless steel structural tubing and its spinner nozzle shall be oriented to propel the spinner in a counterclockwise direction.

  - Hydraulic Requirements: The hydraulic requirements shall be 10-13 gpm (38-49 lpm) @ 5-6 psi (0.3-0.4 bar).