Please refer to PART 1: VORTEX GENERAL CLAUSES for all Play Product construction and installation information.

- Play Product Specifications:

  - Play Product Structure: The Ombrello No1 VOR-7445.2009 shall consist of one (1) Post and one (1) SEEFLOW™ Ombrello Dome. The Post shall be upright and constructed of stainless steel structural tubing with an outside diameter of 3.50" (8.9cm) and a wall thickness of 0.12" (3.0mm). The SEEFLOW™ Ombrello Dome shall have a diameter of 36" (91cm) and shall be constructed of translucent SEEFLOW™ polymer with a center stainless steel spray cap.The SAFESWAP™ anchoring and leveling system shall be used.

  - Overall play product dimensions: The overall height of the Ombrello No1 shall be no less than 122” (310cm) above final grade.

  - Play Product Interactivity: Creates visual interest as water sprays from the top and streams down over the translucent SEEFLOW™ Ombrello Dome.

  - Hydraulic Activity/Components: Eight (8) gentle water streams emitted from the center spray cap shall flow down over the SEEFLOW™ Ombrello Dome.

  - Hydraulic Requirements: The combined hydraulic requirements shall be 5-6 gpm (19-27 lpm) @ 5-6 psi (0.34-0.41 bar).