Please refer to PART 1: VORTEX GENERAL CLAUSES for all Play Product construction and installation information.

- Play Product Specifications:

  - Play Product Structure: The Boat Wheel N°1 VOR 7678.2008 shall be constructed of 304/304L stainless steel structural tubing with an outside diameter of 3½" (8.9cm) and a wall thickness of 0.12" (3mm). A helm wheel shall be assembled to the post with a 3” (7.6cm) solid lead free brass rotational joint allow the wheel to rotate 360° degrees.

  - Overall play product dimensions: The above ground height of the boat wheel shall be no less than 52" (132 cm).

  - Play Product Interactivity: Users can turn the boat wheel to create a spiraling impressive water effect.

  - Hydraulic Activity/Components: The boat wheel shall have three (3)  nozzles to create stream jets.

  - Hydraulic Requirements: The hydraulic requirements shall be 3-6 gpm (11-23 lpm) @10-15 psi (0.7 – 1.0 bar).