Please refer to PART 1: VORTEX GENERAL CLAUSES for all Play Product construction and installation information.

- Play Product Specifications:

  - Play Product Structure: The Astra Nº3 VOR-7699 is constructed into two sections. The bottom section is a fixed structural support for the top and consists of tubing made of 304/304L stainless steel with an outside diameter of 3.50” (8.9cm) and a wall thickness of 0.300” (7.6mm). The upper section consists of a curved vertical post made of 304/304L stainless steel tubing with an outside diameter of 3.50” (8.9cm) and a wall thickness of 0.216” (5.4mm), and a top ring made of 304/304L stainless steel tubing with an outside diameter of 3.50” (8.9cm) and a wall thickness of 0.120” (3.0mm). All bending shall have no joint or ripples. The top section is assembled to the bottom section with a triple stainless-steel bearing system named TWIRLTEC™. The TWIRLTEC™ consists of one (1) soft-touch rotating sphere, fastened at the joint of the posts to tamper resistant hardware. The ring-shaped section is perforated to hold nineteen (19) nozzle housings threaded to accept stainless steel spray nozzles. All nozzles are free of finger entrapment hazards. The SAFESWAP™ anchoring and levelling system shall be used. The TOEGUARD™ will then be added to protect children's toes.

  - Overall play product dimensions: The overall height of the Play Product shall be 102" (259 cm) above the surface with a head clearance of no less than 98" (248 cm).

  - Play Product Interactivity: Nineteen (19) downward water jets create a ring of water for kids to step into or run through. Users can spin the TWIRLTEC™ soft-touch rotating sphere, creating spiraling water jets from the nineteen (19) water nozzles for kids to play with, run through, or play to avoid.

  - Hydraulic Activity/Components: Nineteen (19) downward water jets create a ring of water for kids to step into or run through.

  - Hydraulic Requirements: The hydraulic requirements shall be 10-25 gpm (37-94 lpm) @ 5-8 psi (0.3-0.6 bar).