Please refer to PART 1: VORTEX GENERAL CLAUSES for all Play Product construction and installation information.

- Play Product Specifications:

  - Play Product Structure: The Bamboo Stem Nº2 VOR-7728.2009 shall be constructed of 3.5” (8.9cm) outside diameter stainless steel structural tubing with a wall thickness of 0.120” (3mm). The play unit consists of one fixed, curved, tubular structure with an integrated and recessed nozzle housing welded into the top end, threaded to accept a spray nozzle. A brass spray head nozzle shall be mounted flush with the surface of the nozzle housing.  Attached to the structure, the game has one (1) rounded SEEFLOW™ Polymer panel fastened to the post with tamper resistant hardware. The feature is anchored to the deck surface by means of the SAFESWAP™ Anchor.

  - Overall play product dimensions: It shall have an above ground height of 123" (312cm)

  - Play Product Interactivity: The Bamboo Stem Nº2 creates visual interest to contemplate as a triple-water-jet sprays out from the top end of the post. Users can play with or under the water streams.

  - Hydraulic Activity/Components: The water effect for the Bamboo Stem Nº2 will be three (3) laminar water jets.

  - Hydraulic Requirements: The hydraulic requirements shall be 1-3 gpm (4-11 lpm) @ 7-10 psi (0.5 – 0.7 bar).