Please refer to PART 1: VORTEX GENERAL CLAUSES for all Play Product construction and installation information.

- Play Product Specifications:

  - Play Product Structure: The Pine Tree, VOR-8610, shall be constructed of 304/304L stainless steel structural tubing with an outside diameter of 4½" (11.4cm) and a wall thickness of .120" (3mm) for the trunk. The branches shall be constructed in different sizes of piping, with an outside diameter of 2.375" (6cm) and a wall thickness of .109" (2.8mm), an outside diameter of 1.9" (4.8cm) and a wall thickness of .109" (2.8mm) or an outside diameter of 1.315" (3.3cm) and a wall thickness of .109" (2.8mm). There shall be fifteen (15) rounded SEEFLOW™ Polymer leaves fastened to the branches with tamper resistant hardware. There shall be a threaded nozzle housing recessed into the upper most end cap. The SAFESWAP™ anchoring and leveling system shall be used.

  - Overall play product dimensions: The overall height of the structure shall be no less than 168” (427cm) with a head clearance of 96” (244cm).

  - Play Product Interactivity: The Pine Tree shall create visual interest as water sprays up out the top of the tree and then flows off the leaves.

  - Hydraulic Activity/Components: A lead-free brass cone spray head system mounted on top of the structure shall create either an inverted soft laminar bell spray effect or soft laminar bell spray depending on the nozzle configuration.

  - Hydraulic Requirements: The hydraulic requirements shall be 20-30 gpm (76-114 lpm) @ 10-15 psi (0.7-1.0 bar).